STEELSTAG

TECHNICAL SPECIFICATION PACKAGE  ·  PREMIUM SOLID TEES  ·  SS2026

1.  BRAND & ORDER DETAILS

Brand:  SteelStag

Contact:  hello@steelstag.in

Buyer:  Anjan Mishra  |  anjanimishrait@gmail.com  |  +36 70 198 3287

Approval Address:  1139 Budapest, Rozsnyay utca 14-16, B lph, 1 em, 104 ajto, Hungary

Season:  SS2026 — Initial Launch Run

Styles:  ST-RN-001 (Round Neck)  |  ST-DS-002 (Deep Scoop)  |  ST-VN-003 (V-Neck)

Sizes:  S  /  M  /  L  /  XL

Size Split:  S 15%  |  M 35%  |  L 35%  |  XL 15%

Destination:  India — Direct to Consumer (DTC)

2.  FABRIC SPECIFICATION

Fabric:  100% Combed Cotton — Single Jersey

Yarn Count:  30s Combed Ring Spun

GSM:  125 GSM  (tolerance ±5 GSM)

Construction:  Single Jersey

Dyeing:  Reactive Dyed

Finish:  Light enzyme wash (no heavy silicone finish)

Pre-treatment:  Pre-washed, pre-shrunk fabric

Shrinkage:  Max ±3% after wash (no twisting)

3.  COLOR PALETTE

All colors to be approved on pre-production lab dip before bulk. Pantone TCX references below.

| # | Color Name | Pantone TCX | Style Code | Notes |
| --- | --- | --- | --- | --- |
| 01 | Jet Black | 19-0303 TCX | All styles | Priority — must have |
| 02 | Bright White | 11-0601 TCX | All styles | Priority — must have |
| 03 | Seaport Teal | 17-5126 TCX | All styles | Priority — hero color |
| 04 | Wild Ginger | 15-1157 TCX | All styles | Priority — hero color |
| 05 | Brown Sugar | 18-1048 TCX | All styles |  |

Note: 5 colors across all 3 styles. Provide lab dips for all colors. Priority colors (01–04) to be sampled first.

4.  CONSTRUCTION & STITCH SPECIFICATION

GENERAL

Stitch Density:  12–14 SPI (stitches per inch)

Thread:  Self-colour polyester thread to match each colorway

Seam Type:  Overlock (serger) — all body seams

Side Seam:  Clean overlock — no twisting

NECK FINISH

All styles: Clean-bound neck finish — no rib band. Self-fabric tape, folded and stitched flat. Single needle topstitch 1/8" from edge, self-colour thread. Neck must lie flat against body without rolling or gaping. Reference: white tee reference image provided.

| Style | Neck Construction | Detail |
| --- | --- | --- |
| ST-RN-001 Round Neck | Clean-bound / self-fabric tape | No rib band — folded self-fabric, single needle topstitch 1/8" from edge, lies flat |
| ST-DS-002 Deep Scoop | Clean-bound / self-fabric tape | No rib band — folded self-fabric, single needle topstitch 1/8" from edge, lies flat |
| ST-VN-003 V-Neck | Clean-bound / self-fabric tape | No rib band — mitered V-point, folded self-fabric, single needle topstitch 1/8" from edge |

SLEEVE FINISH

| Style | Sleeve Stitch | Detail |
| --- | --- | --- |
| ST-RN-001 Round Neck | Clean flat hem | Twin needle, 1/4" hem, self-colour |
| ST-DS-002 Deep Scoop | Clean flat hem | Twin needle, 1/4" hem, self-colour |
| ST-VN-003 V-Neck | Clean flat hem | Twin needle, 1/4" hem, self-colour |

HEM

Bottom Hem:  Single needle, 1" hem, self-colour thread

Tolerance:  ±0.5" on all measurements

5.  BODY MEASUREMENTS  (inches)

All measurements in inches. Tolerance ±0.5" unless stated.

| Point of Measure | S | M | L | XL |
| --- | --- | --- | --- | --- |
| HPS Length | 26¾ | 27½ | 28¼ | 29 |
| CB Length (from seam) | 25¾ | 26½ | 27¼ | 28 |
| Shoulder to Shoulder | 17 | 17½ | 18 | 18½ |
| ½ Chest (1" below AH) | 19¼ | 19¾ | 20¼ | 20¾ |
| ½ Waist | 19¼ | 19¾ | 20¼ | 20¾ |
| ½ Sweep (Straight) | 19¼ | 19¾ | 20¼ | 20¾ |
| Armhole Drop from HPS | 10½ | 10¾ | 11 | 11¼ |
| ½ Sleeve Opening | 6 | 6¼ | 6½ | 6¾ |

6.  NECK MEASUREMENTS  (inches)

ROUND NECK — ST-RN-001

| Point of Measure | S | M | L | XL |
| --- | --- | --- | --- | --- |
| Neck Width | 7 | 7¼ | 7½ | 7¾ |
| Front Neck Drop | 2⅞ | 3 | 3⅛ | 3¼ |
| Back Neck Drop | 1 | 1 | 1 | 1 |
| Collar Band Ht | NA | NA | NA | NA |

DEEP SCOOP — ST-DS-002

| Point of Measure | S | M | L | XL |
| --- | --- | --- | --- | --- |
| Neck Width | 8 | 8½ | 8¾ | 9 |
| Front Neck Drop | 4¼ | 4½ | 4⅝ | 4¾ |
| Back Neck Drop | 1 | 1 | 1 | 1 |
| Collar Band Ht | NA | NA | NA | NA |

V-NECK — ST-VN-003

| Point of Measure | S | M | L | XL |
| --- | --- | --- | --- | --- |
| Neck Width | 6¾ | 7 | 7¼ | 7½ |
| Front Neck Drop | 3½ | 3¾ | 4 | 4¼ |
| Back Neck Drop | 1 | 1 | 1 | 1 |
| V-Point Depth | 3½ | 3¾ | 4 | 4¼ |
| Collar Band Ht | NA | NA | NA | NA |

7.  LABEL & BRANDING PLACEMENT

| Label | Type | Position | Notes |
| --- | --- | --- | --- |
| Neck Label | Woven / printed | Centre back, 1" below collar seam | Per size: S / M / L / XL — files attached |
| Wash Care Label | Woven / printed | Left side seam, 3" from bottom hem | File attached |
| Brand Tag | Hang tag | Attached to left side seam with thread | Front + Back — files attached |
| Size Sticker | Printed sticker | On polybag outside | Per size — files attached |

All label artwork files (.png, 300 DPI) attached separately. Do not substitute fonts or colors.

8.  PACKAGING

PRIMARY PACKAGING — KRAFT PAPER STANDUP POUCH

Type:  Kraft paper standup pouch with zipper seal

Size:  32cm × 40cm (fits folded tee — standard retail fold)

Material:  Kraft paper laminated, food-grade inner layer not required

Design:  Two-tone: top 65% natural kraft (#C8A96E), bottom 35% matte black (#1A1A1A)

Logo Print:  "STEELSTAG" centered on kraft section — large, black ink, serif or brand font

Tagline:  "Color. Nothing else." below brand name — smaller, black ink

URL:  "steelstag.in" printed small on dark top band — gold/kraft ink

Closure:  Zipper reseal at top — allows customer to reuse pouch

Finish:  Matte throughout — no gloss lamination

Reference:  Similar to Finn Design (finn-design.de) packaging style

FOLDING & PACKING

Folding:  Standard retail fold — tee folded to approx 28cm × 20cm before pouching

Insert:  Size sticker (provided) affixed on outside of pouch — bottom right corner

Tissue:  Optional: single sheet natural kraft tissue wrap inside pouch

SHIPPING CARTON

Carton:  Export carton — size-wise packing per color

Carton Label:  Brand: SteelStag | Style Code | Color | Size | Qty | PO No.

9.  SAMPLING & APPROVAL PROCESS

| Stage | Requirement | Approval |
| --- | --- | --- |
| Lab Dip | All 5 colors — reactive dyed on approved 125 GSM combed cotton | Written approval required before bulk |
| Fit Sample | 1 pc per style in size M (Seaport Teal or Bright White) | Measurement + visual sign-off |
| Pre-Production | 1 pc per style × all 5 colors | Final approval before cutting |
| Top of Production | 2 pcs per style × per color from first bulk roll | Retained as bulk standard |

Sample Shipment:  All approval samples (lab dips, fit, pre-production) to be shipped to buyer address via DHL or FedEx express. Share AWB tracking number on dispatch. Buyer will provide written sign-off within 5 working days of receipt.

10.  QUALITY & COMPLIANCE

Shrinkage:  Max ±3% (length & width) after 3 washes at 30°C

Color Fastness:  Min Grade 4 — washing, rubbing, perspiration

Pilling:  Min Grade 3–4 after 1000 cycles

GSM Tolerance:  ±5 GSM from approved standard

AQL:  2.5 — inline and final inspection

Certification:  OCS / BCI certificate for cotton preferred — provide mill test report with sample

Shade Consistency:  Shade variation within the same dye lot not to exceed Grade 4 on the Grey Scale (ISO 105-A02). Each dye lot to be accompanied by a shade band (lightest and darkest acceptable piece). Pieces outside the shade band to be rejected.

SteelStag  ·  hello@steelstag.in  ·  steelstag.in  ·  © 2026 SteelStag — Confidential